Remote Control System

Technical Requirements

| Revision | Author (s) | Release Date |
| --- | --- | --- |
| 2.3.2 | FAE Team | 2014, July 24 |
| 2.3.3 | FAE Team | 2015, Jan 14 |
| 2.3.4 | FAE Team | 2015, Mar 12 |
| 2.3.5 | FAE Team | 2015, Apr 23 |
| 2.3.6 | FAE Team | 2015, May 04 |

Contents

1	Objectives	4

2	Environment	5

2.1	Requirements	5

2.2	Network Diagram	5

3	Hardware Requirements	6

3.1	RCS Master Node	6

3.2	RCS Shard	7

3.3	RCS Collector	8

3.4	RCS Anonymizer	9

3.5	RCS Console	10

3.6	Backup	11

3.7	Firewall	12

3.8	Switch	13

4	Network Configuration	14

4.1	VLANs Configuration on Switch	14

4.2	Firewall  Switch Interconnection	15

4.3	Hardware Interconnection Schema	16

4.4	Firewall Rules Setup	17

# Objectives

The present document details requirements needed for RCS installation.

The document includes:

- RCS architecture high level overview

- RCS hardware specifications

- RCS network configuration

# Environment

## Requirements

The following system requirements must be present:

- Rack cabinet

- KVM or separated items (monitor + USB keyboard + USB mouse)

- UPS and power strips to cover all production systems

- Network switch and Firewall as specified in the Hardware Requirements section

- Wired high-speed Internet connection with Static public IP Address; bandwidth of 4Mbit download/1 Mbit upload is recommended

## Network Diagram

# Hardware Requirements

Software and relative hardware quantities of Master Node, Shards, Collectors, Anonymizers and Consoles are indicated in the commercial offer. One backup unit, although strongly suggested, is optional.

## RCS Master Node

### System Requirements

The following must be present:

- 96 GB of RAM minimum

- 2 x 146GB SAS HD RAID1 (for O.S.)

- 4 x 600GB SAS HD RAID 10 (for data)

- Windows Server 2008 R2 SP1 Enterprise Edition 64 Bit (English)

### Disk Configuration

The following table details how disks must be configured:

| Qty | Disk | RAID | Partitioning | Notes |
| --- | --- | --- | --- | --- |
| 2 | 146 GB | RAID 1 | NTFS, single partition | Install O.S. here |
| 4 | 600 GB | RAID 10 | NTFS, single partition | Mount as C:\RCS |

### Suggested Hardware Specifications

Below you can find a recommended hardware configuration for RCS Master Node.

Dell PowerEdge R720

CPU : Intel Xeon E5-2660 2.20Ghz, 20MB Cache

RAM : 12 x 8GB RDIMM, 1600Mhz

HD (OS) : 2 x 146GB SAS 6Gbps 15k 2.5” HD Hot Plug (RAID1)

HD (Data) : 4 x 600GB SAS 6Gbps 10k 2.5” HD Hot Plug (RAID10)

RAID : PERC H710p Integrated RAID Controller

Network : Broadcom 5720 QP 1Gb Network Card

Optical : 16X DVD+/-RW Drive SATA

### Additional Configurations

Enable the NTP Synchronization towards the NTP server on the RCS Collector.

You would need at most one (1) Master Node unit.

Specific HBA card is required depending on the SAN or DAS selected.

## RCS Shard

This is an optional module and can be skipped according to customer’s license.

### System Requirements

The following must be present according to the purchased license:

- 96 GB of RAM minimum

- 2 x 146GB SAS HD RAID1 (for O.S.)

- 4 x 600GB SAS HD RAID 10 (for data)

- Windows Server 2008 R2 SP1 Enterprise Edition 64 Bit (English)

### Disk Configuration

The following table details how disks must be configured:

| Qty | Disk | RAID | Partitioning | Notes |
| --- | --- | --- | --- | --- |
| 2 | 146 GB | RAID 1 | NTFS, single partition | Install O.S. here |
| 4 | 600 GB | RAID 10 | NTFS, single partition | Mount as C:\RCS |

### Suggested Hardware Specification

Below there is a recommended hardware configuration for RCS Shard.

Dell PowerEdge R720

CPU : Intel Xeon E5-2660 2.20Ghz, 20MB Cache

RAM : 12 x 8GB RDIMM, 1600Mhz

HD (OS) : 2 x 146GB SAS 6Gbps 15k 2.5” HD Hot Plug (RAID1)

HD (Data) : 4 x 600GB SAS 6Gbps 10k 2.5” HD Hot Plug (RAID10)

RAID : PERC H710p Integrated RAID Controller

Network : 2 x Broadcom 5720 QP 1Gb Network Card

Optical : 16X DVD+/-RW Drive SATA

### Additional Configurations

Enable the NTP Synchronization towards the NTP server on the RCS Collector.

Refer to the commercial offer for the exact number of Shards you need. 
In case of doubt please refer to your sales representative.

## RCS Collector

### System Requirements

The following must be present:

- 16 GB of RAM minimum

- 2 x 300GB SAS HD RAID1 (for O.S. and data)

- Windows Server 2008 R2 SP1 Standard Edition (or above) 64 Bit (English)

### Disk Configuration

The following table details how disks must be configured:

| Qty | Disk | RAID | Partitioning | Notes |
| --- | --- | --- | --- | --- |
| 2 | 300 GB | RAID 1 | NTFS, single partition | Install O.S. here |

### Suggested Hardware Specifications

Below you can find a recommended hardware configuration for RCS Collector.

Dell PowerEdge R210 II

CPU : Intel Xeon E3-1230 3.20Ghz, 8MB Cache

RAM : 2 x 8GB DDR3, 1333Mhz

HD (OS) : 2 x 300GB SAS 6Gbps 15k 2.5” HD Hot Plug (RAID1)

RAID : PERC H200 RAID Controller

Network : 2 x Broadcom 5720 QP 1Gb Network Card

Optical : 16X DVD+/-RW Drive SATA

### Additional Configurations

Enable the NTP Synchronization towards the closest public NTP server.

Refer to the commercial offer for the exact number of Collectors you need. 
In case of doubt please refer to your sales representative.

## RCS Anonymizer

### System Requirements

The following must be present:

- 256 MB of RAM minimum

- 10GB HD

- Linux CentOS 6 32 Bit

- Static public IP address

- 2 Mbit/s Internet connection

Due to company policies and to protect customer’s confidentiality requirements, Hacking Team is not allowed to provide accounts on VPS services.

### Suggested VPS List

The following table list examples of possible VPS providers:

| Name | Web site | Locations |
| --- | --- | --- |
| Linode | http://www.linode.com | USA and many other locations |
| Host Europe | http://www.hosteurope.de | Germany and other locations |

Refer to the commercial offer for the exact number of Anonymizers you need. 
In case of doubt please refer to your sales representative.

## RCS Console

A VPN connection is suggested when connecting to RCS Master Node from external network.

### System Requirements

The following must be present:

- 8 GB of RAM minimum

- 320GB SATA HD

- Windows or OS X

- Display capable of 1280x800 pixel minimum resolution

### Suggested Hardware Specifications

Below you can find a recommended hardware configuration for RCS Console.

Dell Latitude 15 – Series 3000

CPU : Intel Core i3-3120M 2.50Ghz, 3MB Cache

RAM : 8GB DDR3, 1600Mhz

HD : 1 x 320GB SATA 7.2k 2.5” HD

Video : Intel HD Graphics 4000

Network : 1 x 1Gb Network Card, 1 x Dell Wireless 1901 802.11 a/b/g/n

Optical : 8X DVD+/-RW Drive SATA

### Additional Configurations

Adobe Air runtime must be installed on the system.

Refer to the commercial offer for the exact number of Consoles you need. 
In case of doubt please refer to your sales representative.

## Backup

The backup unit is a SAN (Storage Area Network) or a DAS (Direct-Attached Storage) device that is responsible for all RCS data backup.

Network share drives i.e. SMB, NFS are not supported.

A backup unit is strongly suggested.

### System Requirements

The following must be present:

- 6 x 1TB SAS HD RAID6 (for data)

### Disk Configuration

The following table details how disks must be configured:

| Qty | Disk | RAID | Partitioning | Notes |
| --- | --- | --- | --- | --- |
| 6 | 1 TB | RAID 6 | NTFS, single partition | Mount as Z:\ |

### Suggested Hardware Specifications

Below you can find a recommended hardware configuration for backup unit.

DELL PowerVault MD3200i

HD : 6 x 1TB SAS 6Gbps 7.2k HD Hot Plug

## Firewall

Availability of the firewall is mandatory for the delivery.

### System Requirements

The following must be present:

- Support for VPN connection client to site (SSL or IPSEC)

- Stateful throughput of 1 Gbps

- IMIX performance of 235 Mbps

- Maximum connections of 225000

- VPN throughput of 300 Mbps

The availability of a firewall with such characteristics dedicated to the RCS installation is compulsory for the completion of the installation.

### Suggested Hardware Specifications

Below you can find a recommended hardware configuration for firewall.

SonicWall NSA 3600 Network Security Appliance

IPSEC VPN Connections Client to Site: Up to 1000

Firewall inspection throughput: 3.4 Gbps

IMIX Performance: 900 Mbps

Maximum Connections: 325000

VPN Throughput: 1.5 Gbps

Or

SonicWall NSA 2600 Network Security Appliance

IPSEC VPN Connections Client to Site: Up to 250

Firewall inspection throughput: 1.9 Gbps

IMIX Performance: 600 Mbps

Maximum Connections: 225000

VPN Throughput: 1.1 Gbps

## Switch

### System Requirements

The following must be present:

- 24 ports

- Support for 10/100/1000 Mbps

### Suggested Hardware Specifications

Below you can find a recommended hardware configuration for the switch.

Dell PowerConnect 2800

Ports : 24 at least

Speed: 10/100/1000 Mbps

# Network Configuration

## VLANs Configuration on Switch

The RCS environment requires 5 VLANs on a switch.

These VLANs create different logical LAN for each RCS component and for devices management.

On the switch you can create there VLANs:

- Backend VLAN

- Collector VLAN

- Console VLAN

- Firewall Management VLAN

- Switch Management VLAN

The assigned ports on the switch for each VLAN could be 2 or more, depending on the architecture.

## Firewall  Switch Interconnection

The firewall is used to regulate communication between VLANs.

Five zones are configured on the firewall:

- Backend VLAN

- Collector VLAN

- Console VLAN

- Firewall Management VLAN

- External VLAN (Internet)

Zones on the firewall and VLANs on the switch must be connected according to the picture below.

## Hardware Interconnection Schema

Following is represented the whole system architecture with its interconnections. As described in the picture, final infrastructure may include additional RCS Collectors and RCS Shards.

## Firewall Rules Setup

The following rules must be implemented on the firewall to allow RCS works correctly.

Table’s colors reflect the colors used in previous pictures.

| Source | Destination | Service | Protocol | Port |
| --- | --- | --- | --- | --- |
| Backend | Any | DNS | UDP | 53 |
| Backend | Any | NTP | UDP | 123 |
| Backend | Collector | HTTP | TCP | 80 |
| Console | Any | HTTPS | TCP | 443 |
| Console | Any | HTTP | TCP | 80 |
| Console | Any | DNS | UDP | 53 |
| Console | Any | ICMP | ICMP |  |
| Console | Collector | RDP | TCP | 3389 |
| Console | Backend | RDP | TCP | 3389 |
| Console | Backend | HTTPS | TCP | 443 |
| Console | Backend | TCP_444 | TCP | 444 |
| Collector | Any | DNS | UDP | 53 |
| Collector | Any | HTTP | TCP | 80 |
| Collector | Any | HTTPS | TCP | 443 |
| Collector | Any | NTP | UDP | 123 |
| Collector | TNI | HTTPS | TCP | 443 |
| Collector | Backend | HTTPS | TCP | 443 |
| Collector | Backend | TCP_442 | TCP | 442 |
| Anonymizer(s) | Collector | HTTP | TCP | 80 |